# Project Specifications & Standards

This document outlines the standard configuration values, parameters, and time metrics used across the project to maintain consistency across all modules.

# 1. Time & Sampling Specifications

| Parameter | Symbol | Default Value | Tunability | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| Simulation Time Step | Δt (DT) | 0.2 | Fixed / Standard | seconds (s) | Physical duration of each simulation and prediction step (5 Hz). |
| Prediction Horizon | N (horizon) | 20 | Tunable | steps | Number of future steps predicted by nominal model & MPC (4.0 s lookahead). |
| Horizon Duration | T_horizon | 4.0 | Derived (N × Δt) | seconds (s) | Total lookahead time span. |
| Historical Sample Duration | T_sample | 4.0 | Derived | seconds (s) | Spans 4.0 s (21 timesteps: t=0 initial + 20 future steps). |
| Historical Sample Size | S | 100 | Tunable | samples | Total historical trajectory samples collected into offline dataset. |
| Control Update Interval | CONTROL_EVERY | 1 or 5 | Tunable | steps | Receding horizon solver update frequency. |

# 2. Pedestrian Population Specifications

| Phase / Context | Default Count | Configurable / Switchable | Description / Purpose |
| --- | --- | --- | --- |
| Training Phase (Historical Data Collection) | 10 pedestrians | Yes (tunable parameter) | 10 pedestrians simulated in the environment to collect trajectory batches until S=100 samples are gathered. |
| Testing / Real-Time Phase (Live DR-MPC Simulation) | 5 pedestrians | Yes (tunable parameter) | 5 active pedestrians in the live environment for real-time threat assessment and vehicle control. |
| Live LiDAR Detections | D ≤ 5 pedestrians | Dynamic (0 to 5) | Number of pedestrians within 12m LiDAR radius at any control step. |

# 3. Environment, Vehicle & Sensor Specifications

| Component | Parameter | Value | Unit | Notes |
| --- | --- | --- | --- | --- |
| World | Width × Height | 120 × 40 | meters (m) | Continuous 2D road simulation domain with periodic x-wrapping. |
| Road | Centerline Y / Width | 20 / 12 | meters (m) | Road spans y in [14, 26] m. Sidewalks in [0, 14] m and [26, 40] m. |
| Vehicle | Length × Width | 3.0 × 2.0 | meters (m) | Compact ego vehicle with center mounted sensor. |
| Vehicle | Desired Speed (v_des) | 8.0 | m/s (~28.8 km/h) | Target longitudinal cruising speed. |
| Vehicle | Speed Limits [vmin, vmax] | [0.0, 15.0] | m/s | Operating speed bounds. |
| Vehicle | Accel Limits [amin, amax] | [-5.0, +3.0] | m/s² | Max braking (-5.0) and acceleration (+3.0). |
| LiDAR | Detection Radius (R) | 12.0 | meters (m) | Circular range sensing around vehicle center. |